K12

Kawasaki ZX-10R 2006-2007

Fitting kit left:		     		          Fitting kit right:

| ID | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Steel adapter | 554 | 1 |
| 3 | Bolt M10X70 | 851 | 1 |
| 4 | Bolt M10x70x1,25 | 898 | 1 |
| 5 | Dural.adapter d=30 | 11 | 1 |
| 6 | Dural.cylinder d=14 | 114 | 1 |
| 7 | Washer M10 spring | 801 | 1 |
| 8 | Tape 3M |  | 1 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Steel adapter | 553 | 1 |
| 3 | Bolt M10x70 | 851 | 1 |
| 4 | Bolt M10x80x1,25 | 920 | 1 |
| 5 | Dural.adapter d=30 | 11 | 1 |
| 6 | Washer M10 spring | 801 | 1 |
| 7 | Tape 3M |  | 1 |

Pad for expansion bottle.

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 4,7 2 9 | 6 Dural.cylinder d=6 | 2 4,6 115 | 2 |

5

5

3

1

9

7,8

3

1

Assembly make here:

Insert pad under the expansion bottle.

Assembly left side:

- Take off fairing and bolt from frame M10.

- On the adapter ID.2 stick 3M tape in point where is adapter touching with frame.

- Through the adapter insert bolt with pad ID.4 and ID.7 Put dural.cylinder ID.6 on bolt from back side. Between the adapter and frame.

- This kit mounted on the frame.

- Into the protector insert steel box ID.1. Under the protector put dural.adapter. ID.5. This kit mounted on the steel adapter. Use bolt ID.3

- Put fairing back.

Assembly right side:

- Take off fairing and bolt from frame M10.

- On the adapter ID.2 stick 3M tape in point where is adapter touching with frame.

- Through the adapter insert bolt with pad ID.4 and ID.6.

- This kit mounted on the frame.

- Into the protector insert steel box ID.1. Under the protector put dural.adapter. ID.5. This kit mounted on the steel adapter. Use bolt ID.3

- Put fairing back.

Warning: We recommend use the glue of all threads. Medium strong.