K13SD

Kawasaki Z1000 2007-2009

| Kit for the left side |  |  |  | Kit for the right side |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 2 | SLD adapter | 537 | 1 |  |  | 2 | SLD adapter | 537 | 1 |  |
| 3 | Bolt M8x60mm | 838 | 2 |  |  | 3 | Bolt M8x60mm | 838 | 2 |  |
| 4 | Dural. cylinder 14mm | 18 | 2 |  |  | 4 | Dural. cylinder 14mm | 18 | 2 |  |
| 5 | Dural. adapter | 547 | 1 |  |  | 5 | Dural. adapter | 547 | 1 |  |
| 6 | Plastic cover | X5 | 2 |  |  | 6 | Plastic cover | X5 | 2 |  |
| 7 | 3M tape | 3M | 1 |  |  | 7 | 3M tape | 3M | 1 |  |
| 8 | Dural. cylinder 30mm | 11 | 1 |  |  | 8 | Dural. cylinder 15mm | 9 | 1 |  |
| 9 | Bolt M8 | 815 | 2 |  |  | 9 | Bolt M8 | 815 | 2 |  |
| 10 | Washer M8 (stringed) | 800 | 2 |  |  | 10 | Washer M8 (stringed) | 800 | 2 |  |
| 11 | Bolt M10x50 | 849 | 1 |  |  | 11 | Bolt M10x35 | 847 | 1 |  |

Instructions:

- Remove original bolt M8 from the frame of motorcycle.

- Mount dural. adapter (ID5) and dural cylinder (ID4) using bolts M8 (ID3) on the frame of motorcycle.

- Cover bolts with covers ID6.

- Mount SLD adapter ID2 and dural. cylinder ID8 on the dural. adapter using bolt ID11.

- In the end mount slider using bolts ID9 and washers ID10.

We recommend to use the glue on all threads (medium strong).