K15SDM

Kawasaki ZX-10R 2008-2010

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 1 | Steel adapter | 555 | 1 |  |  | 1 | Steel adapter | 556 | 1 |  |
| 2 | SLDM adapter | 537B | 1 |  |  | 2 | SLDM adapter | 537B | 1 |  |
| 3 | Bolt M10x50x1,25 mm | 921 | 1 |  |  | 3 | Bolt M10x50x1,25 mm | 921 | 1 |  |
| 4 | Bolt M10x90 mm | 880 | 1 |  |  | 4 | Bolt M10x50 mm | 849 | 1 |  |
| 5 | Washer M10 (stringed) | 801 | 1 |  |  | 5 | Washer M10 (stringed) | 801 | 1 |  |
| 6 | Washer M10 (flat) | 804 | 1 |  |  | 6 | Washer M10 (flat) | 804 | 1 |  |
| 7 | Dural. cylinder d=10mm | 113 | 1 |  |  | 7 | Dural. cylinder d=4mm | 112 | 1 |  |
| 8 | Dural. cylinder d=60mm | 14 | 1 |  |  | 8 | Dural. cylinder d=40mm | 12 | 1 |  |
| - | Rod AL16(M10)x410x30x30 |  | 1 |  |  | 9 | Bolt M10x30mm | 846 | 1 |  |
| 10 | Tape 3M |  |  |  |  | 10 | Tape 3M |  |  |  |
| 11 | Bolt M6x20 | 1024 | 2 |  |  | 11 | Bolt M6x20 | 1024 | 2 |  |
| 12 | Washer M6 | 1023 | 2 |  |  | 12 | Washer M6 | 1023 | 2 |  |

8

2

2

9

4

4

8

1

1

11+12

3+5+6

3+5+6

7

7

11+12

10

Instructions:

- Remove fairing of motorcycle.

- Remove original bolts M10 from the frame of motorcycle.

- Stick 3M tape on the frame of motorcycle.

- Fit steel adapter (ID1) on the left side using bolt (ID3). Place washers (ID5 and ID6) and cylinder (ID7) on the bolt.

- Fit steel adapter on the right side (ID1) using bolt (ID3). Place washers (ID5 and ID6) and cylinder (ID7) on the bolt.

- Fit connecting rod on the right side adapter using bolt (ID9).

- Fit kit for the left side protector. Put adapter (ID2) and cylinder ID8 on the bolt ID4. Mount this kit into thread of connecting rod.

- Now fit kit for the right side protector. Put SLDM adapter (ID2) and cylinder ID8 on the bolt ID4. Mount this kit into the thread of steel adapter ID1.

- In the end fit protectors using two bolts M6 and washers M6 (ID11 and ID12).

We recommend to use the glue on all threads (medium strong).