K16SD

Kawasaki Versys 650 2006-

| LEFT SIDE KIT |  |  |  |  |  | RIGHT SIDE KIT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID |  | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 1 |  | Adapter SLD | 537 | 1 |  |  | 1 | Adapter SLD | 537 | 1 |  |
| 2 |  | Nut M8 | 813 | 1 |  |  | 2 | Nut M8 | 813 | 2 |  |
| 3 |  | Washer M8 | 802 | 1 |  |  | 3 | Washer M8 | 802 | 2 |  |
| 4 |  | Dural. cylinder 50mm | 209 | 1 |  |  | 4 | Dural.váleček d=30m | 170 | 1 |  |
| 5 |  | Bolt M12x100 | 986 | 1 |  |  | 5 | Bolt M12x80 | 956 | 1 |  |
| 6 |  | Dural adapter | 544 | 1 |  |  | 6 | Dural. adapter | 543 | 1 |  |
| 7 |  | Bolt M10x100x1,25mm | 892 | 1 |  |  | 7 | Bolt M10x110x1,25mm | 928 | 1 |  |
| 8 |  | Bolt M8x40mm | 828 | 1 |  |  | 8 | Bolt M8x40 | 828 | 2 |  |
| 9 |  | Dural. cylinder 41mm | 36 | 1 |  |  | 9 | dural. cylinder 61mm | 37 | 1 |  |
| 10 |  | Dural. SLD adapter 15mm | DV149 | 1 |  |  | 10 | Dural. SLD adapter | DV149 | 1 |  |
| 11 |  | Bolt M8x25 | 815 | 2 |  |  | 11 | Bolt M8x25 | 815 | 2 |  |
| 12 |  | Washer M8 | 800 | 2 |  |  | 12 | Washer M8 | 800 | 2 |  |
| 13 |  | Dural. rod AL20(M12)x314x30x30 |  | 1 |  |  |  |  |  |  |  |

INSTRUCTIONS

- Remove side fairing and unbolt bolts M10 and M8.

- Fit dural adapter ID6 on the left side using bolt ID7 and ID8, use safety nut ID2 and washer ID3.

- Insert dural. cylinder ID9 between frame and motorcycle.

- Fit steel adapter ID6 on the right side using bolt ID7 and ID8, use nut ID2 and washer ID3.

- Insert dural. cylinder ID9 between frame and motorcycle.

- Put bolt ID5 through adapter ID1, insert adapter ID10 and cylinder ID4 under steel adapter.

- Mount this kit into dural. rod ID13. Place this rod before engine, between adapters for protectors.

- Fit SLD adapter ID1 and adapter ID10 and cylinder ID4 using bolt ID5 into dural rod ID13.

- Fit slider using three bolts ID11.

- Fit cover using two bolts ID12.

We recommend to use safety glue on all threads.

1

1

6

6

2,3

8

2,3

8

9

9

7

5

7

5

4

4

10

10

Old type of protector