K16S

Kawasaki Versys 650 2006-

| Fitting kit for left side |  |  |  |  | Fitting kit for right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No | Qty |  |  | ID | Description | Cat.No | Qty |  |
| 1 | slider RDmoto | SL | 1 |  |  | 1 | slider RDmoto | SL | 1 |  |
| 2 | Cap | SLcap.. | 1 |  |  | 2 | Cap | SLcap.. | 1 |  |
| 3 | Adapter for slider | DV-314 | 1 |  |  | 3 | Adapter for slider | DV-314 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |
| 6 | Dural.adapte | 544 | 1 |  |  | 6 | Dural.adapte | 543 | 1 |  |
| 7 | Bolt M10x100x1,25mm | 892 | 1 |  |  | 7 | Bolt M10x110x1,25mm | 928 | 1 |  |
| 8 | Bolt M8x40mm | 828 | 1 |  |  | 8 | Bolt M8x40 | 828 | 2 |  |
| 9 | dural. cylinder d=41mm | 36 | 1 |  |  | 9 | dural.cylinder d=61mm | 37 | 1 |  |
| 10 | Bolt M12x70mm | 977 | 1 |  |  | 10 | Bolt M12x50mm | 869 | 1 |  |
| 11 | Nut M8 | 813 | 1 |  |  | 11 | Nut M8 | 813 | 2 |  |
| 12 | dural.bar AL20(M12)x314x30x30 |  | 1 |  |  | 12 | Dural.cylinder d=15mm | 148 | 1 |  |
| 13 | Dural.cylinderd=30m | 170 | 1 |  |  |  |  |  |  |  |

Instruction to assembly:

- Dismantle original bolts from frame M10 a M8

- On the left side gripping dural.adapter ID.6 use bolt M10x100 ID.7, and bolt M8x40 ID.8, use nut  M8 ID.11

- Insert al.cylinder d=41mm ID.9 between the frame and dural adapter

- Gripping dural.adapter on the right side ID.6 use bolt M10x110 ID.7 and bolt M8x40 ID.8, use nuts  M8 ID.11

- Insert dural.cylinder between the frame and adapter ID.9 d=61mm

- Use bolt M12x70 ID.10 and put on it dural.adapter for slider ID.3 and dural.cylinder d=30mm ID.13

- Take the dural. bar and insert between the dural. adapters ID.12

- Into the this bar mount fitting kit for sliders.

- Instruction for assembly right side , use bolt  M12x50 ID.10 and put on it dural.adapter  ID.3 and dural.cylinder  d=15mm ID.12 andthis kit mounting on the dural.adapter.

- Use bolts M6x25 ID.4 and make to assembly sliders.

- Insert caps ID. 2 into the sliders and use bolts ID.5

Warning- use the glue of all threads. Medium strong.