K20

Kawasaki ZX-6R 2000-2002

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M10 | X1 | 1 |  |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M10x100x1,25mm | 892 | 1 |  |  | 2 | Bolt M10x100x1,25mm | 892 | 1 |  |
| 3 | Dural. cylinder d=60mm | 6 | 1 |  |  | 3 | Dural. cylinder d=40mm | 4 | 1 |  |
| 4 | Bolt M10x110mm | 879 | 1 |  |  | 4 | Bolt M10x80mm | 852 | 1 |  |
| 5 | Pad M10 (plane) | 804 | 3 |  |  | 5 | Bolt M10x30mm | 882 | 1 |  |
| 6 | Steel adapter | 607 | 1 |  |  | 6 | Bolt M8x60mm | 839 | 1 |  |
| 7 | Duralumin bar AL16(M10)x385x30x30 |  | 1 |  |  | 7 | Pad M10 (plane) | 804 | 2 |  |
| 8 | 3M tape |  | 1 |  |  | 8 | Steel adapter | 608 | 1 |  |
|  |  |  |  |  |  | 9 | Duralumin cylinder d=43mm | 254 | 1 |  |
|  |  |  |  |  |  | 10 | Duralumin cylinder d=4,5mm | 298 | 1 |  |
|  |  |  |  |  |  | 11 | Nut M8 (safety) | 809 | 1 |  |
|  |  |  |  |  |  | 12 | 3M tape |  | 1 |  |

Installation:

- remove the complete side cowl

- remove the original bolts M10

- use tape 3M for mounting of the steel adapters for the restricted space between adapter and frame of the motorcycle

- use the bolt (No. 2) for fixing steel adapters (No. 6) on the left side, give the pad (No. 5) under the head of the bolt M10, the next two pieces of pads (No. 5) give on bolt M10 the other side of steel adapter (this pads determine the will between the frame of motorcycle and the steel adapter)

- tighten the steel adapter on the left side lightly

- on the right side use the bolts (No. 2 and No. 6) for mounting of the steel adapter (No. 8). The other side of adapter give on the bolt (No. 6) the duralumin cylinder (No. 9) and on the bolt (No. 2) give the duralumin cylinder (No. 10) too. Give it into the frame of motorcycle and tighten lightly the bolt M10. Give the pad M10 (No. 7) under the head of the bolt. Ensure the bolt M8 with the nut (No. 11). Tighten all only lightly now.

- take the duralumin bar (No. 7, Cat. No. AL2), use the bolt (No. 5) with the pad (No. 7) for mounting of the bar on the steel adapter on the right side

- now you can mount the crash protector on the left side, take the crash protector and give into the protector the steel box (No. 1) together with the bolt (No. 4), the other side of protector give on bolt the duralumin cylinder (No. 6) and this all set put through the steel adapter and mount into duralumin bar (No. 7, Cat. No. AL2)

- take the crash protector for the right side of the motorcycle, give the steel box (No. 1) and the bolt (No. 4)  into the protector, the other side of the protector give on bolt the duralumin cylinder (No. 3) and all this set mount into the steel adapter

- tighten the complete fitting kit with the crash protectors, mount the side cowl of the motorcycle back

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).

FRONT PART OF ENGINE

bolt M10x110mm + steel box M10 + duralumin cylinder d=60mm

duralumin bar d=385mm

bolt M10x30mm + pad M10 (plane)

steel adapter

bolt M10x80mm + steel box M10 + duralumin cylinder d=40mm

bolt M8x60mm + duralumin cylinder d=48mm + nut M8 (safety)

bolt M10x100x1,25 mm + three pads M10 (plane)

bolt M10x100x1,25mm + duralumin pad d=4,5mm + pad M0 (plane)

steel adapter