K22

Kawasaki ZX-9R 2000-2001

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M10 | X1 | 1 |  |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M10x80x1,25mm | 920 | 1 |  |  | 2 | Bolt M10x100x1,25mm | 892 | 1 |  |
| 3 | Dural. cylinder d=50mm | 5 | 1 |  |  | 3 | Dural. cylinder d=50mm | 5 | 1 |  |
| 4 | Bolt M10x100mm | 885 | 1 |  |  | 4 | Bolt M10x100mm | 885 | 1 |  |
| 5 | Pad M10 (plane) | 804 | 2 |  |  | 5 | Bolt M8x60mm | 838 | 1 |  |
| 6 | Steel adapter | 534 | 1 |  |  | 6 | Pad M8 (plane) | 802 | 2 |  |
| 7 | Duralumin bar AL16(M10)x392x30x30 |  | 1 |  |  | 7 | Pad M10 (plane) | 804 | 1 |  |
| 8 | Pad (plane) M6 | 947 | 4 |  |  | 8 | Steel adapter | 536 | 1 |  |
| 9 | 3M tape |  | 1 |  |  | 9 | Duralumin cylinder d=18mm | 299 | 1 |  |
|  |  |  |  |  |  | 10 | Nut M8 (safety) | 809 | 1 |  |
|  |  |  |  |  |  | 11 | 3M tape |  | 1 |  |

Installation:

- remove the complete side cowl

- remove the original bolts M10

- use tape 3M for mounting of the steel adapters for the restricted space between adapter and frame of the motorcycle

- use the bolt (No. 2) for fixing steel adapters (No. 6) on the left side, give the pad (No. 5) under the head of the bolt M10, the next piece of pads (No. 5) give on bolt M10 the other side of steel adapter (this pads determine the will between the frame of motorcycle and the steel adapter)

- tighten the steel adapter on the left side lightly

- on the right side use the bolts (No. 2 and No. 5) for mounting of the steel adapter (No. 8). The other side of adapter give on the bolt (No. 5) with the pad (No. 6) the duralumin cylinder (No. 9). Give it into the frame of motorcycle and tighten lightly the bolt M10. Give the pad M10 (No. 7) under the head of the bolt. Ensure the bolt M8 with the nut (No. 10) with the pad (No. 6). Tighten all only lightly now.

- now you can mount the crash protector on the left side, take the crash protector and give into the protector the steel box (No. 1) together with the bolt (No. 4), the other side of protector give on bolt the duralumin cylinder (No. 3) and this all set put through the steel adapter and mount into duralumin bar (No. 7, Cat. No. AL3)

- take the crash protector for the right side of the motorcycle, give the steel box (No. 1) and the bolt (No. 4)  into the protector, the other side of the protector give on bolt the duralumin cylinder (No. 3) and and this all set put through the steel adapter and mount into duralumin bar (No. 7, Cat. No. AL3)

- tighten the complete fitting kit with the crash protectors, mount the side cowl of the motorcycle back

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).

FRONT PART OF ENGINE

bolt M10x100mm + steel box M10 + duralumin cylinder d=50mm

duralumin bar d=392mm

bolt M10x100mm + steel box M10 + duralumin cylinder d=50mm

steel adapter

bolt M8x60mm + two pads M8 (ploché) + duralumin cylinder d=18mm + nut M8

bolt M10x80x1,25 mm + three pads M10 (plane)

bolt M10x100x1,25mm + pad M0 (plane)

steel adapter