K23SDM

Kawasaki ER-6N 2009-2011

| LEFT SIDE KIT |  |  |  |  | RIGHT SIDE KIT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |  |
| 1 | Adapter SLDM | 537/B | 1 |  |  | 1 | Adapter SLDM | 537/B | 1 |  |  |
| 2 | Bolt M10x130x1,25mm | 905 | 1 |  |  | 2 | Bolt M10x120x1,25mm | 895 | 1 |  |  |
| 3 | Bolt M8x60mm | 838 | 1 |  |  | 3 | Bolt M8x40mm | 835 | 2 |  |  |
| 4 | Bolt M10x60mm | 850 | 1 |  |  | 4 | Bolt M10x70mm | 851 | 1 |  |  |
| 5 | Dural. cylinder 40mm | DV12 | 1 |  |  | 5 | Dural. cylinder 50mm | DV13 | 1 |  |  |
| 6 | Dural. cylinder 63mm | 118 | 1 |  |  | 6 | Dural. cylinder 4,5mm | 116 | 1 |  |  |
| 7 | Dural. cylinder 22mm | 117 | 1 |  |  | 7 | Dural. adapter | 527 | 1 |  |  |
| 8 | Dural. adapter | 528 | 1 |  |  | 8 | Washer M10 (flat) | 804 | 1 |  |  |
| 9 | Washer M10 (flat) | 804 | 1 |  |  | 9 | Bolt M6x20 | 1024 | 2 |  |  |
| 10 | Bolt M6x20 | 1024 | 2 |  |  | 10 | Washer M6 | 1023 | 2 |  |  |
| 11 | Washer M6 | 1023 | 2 |  |  |  |  |  |  |  |  |

LEFT SIDE INSTRUCTIONS

- Dismantle side fairing and bolts from the frame.

- Assembly fitting kit according to the picture above (page 1).

- Fit this kit into the frame of motorcycle.

- Fit SLD adapter ID1 and dural cylinder ID5 into the fitting kit using bolt ID4.

- Fit slider using bolts ID10 and washers ID11.

We recommend to use safety glue on all threads (medium strong).

RIGHT SIDE INSTRUCTIONS

- Dismantle side fairing and bolts from the frame.

- Assembly fitting kit according to the picture above (page 1).

- Fit this kit into the frame of motorcycle.

- Fit SLD adapter ID1 and dural cylinder ID5 into the fitting kit using bolt ID4.

- Fit slider using bolts ID9 and washers ID10.

We recommend to use safety glue on all threads (medium strong).