K24SD

Kawasaki ER-6F 2009-2011

Mounting Kit Left Side:		     		Mounting Kit Right Side:

| ID | Description | Cat. No. | Pcs. |
| --- | --- | --- | --- |
| 1 | Alu. Roller d=60mm | DV14 | 1 |
| 2 | Dural. Adapter SLD | 537 | 1 |
| 3 | Bolt M10x85 | 981 | 1 |
| 4 | Bolt M8x80 | 843 | 2 |
| 5 | Washer M8 (Spring) | 800 | 2 |
| 6 | Dural. Roller d=56 | 222 | 1 |
| 7 | Dural. Roller | 220 | 1 |
| 8 | Bolt M6x60 | 872 | 1 |
| 9 | Nut M6 | 948 | 1 |
| 10 | Washer M6 | 947 | 2 |
| 11 | 3M Tape | - | 1 |
| 12 | Steel Adapter | 532 | 1 |
| 13 | Bolt M8x25 | 815 | 2 |
| 14 | Washer M8 | 800 | 2 |

| ID | Description | Cat. No. | Pcs. |
| --- | --- | --- | --- |
| 1 | Alu. Roller d=70mm | DV15 | 1 |
| 2 | Dural. Adapter SLD | 537 | 1 |
| 3 | Bolt M10x95 | 822 | 1 |
| 4 | Bolt M8x70 | 841 | 2 |
| 5 | Washer M8 (Spring) | 800 | 2 |
| 6 | Dural. Roller d=36,2 | 221 | 2 |
| 7 | Bolt M6x50 | 867 | 1 |
| 8 | Nut M6 | 948 | 1 |
| 9 | Washer M6 | 947 | 2 |
| 10 | 3M Tape | - | 1 |
| 11 | Steel Adapter | 533 | 1 |
| 12 | Bolt M8x25 | 815 | 2 |
| 13 | Washer M8 | 800 | 2 |

| 13 | Strut | X42 | 1 |
| --- | --- | --- | --- |
| 14 | Washer M8 | 800 | 2 |
| 15 | Bolt M8x20 | 824 | 2 |

1

3

2

5

4

14, 15

14, 15

7

6

8

4

4

6

6

Use the rollers included in this kit instead of the crossed out rollers in the attached pictures.

Mounting Instructions, Left Side:

- Remove the motorcycle fairing.

- At the place of a steel holder, remove the original bolts from motorcycle.

- Insert ID.4 bolts into the steel adapter ID.12 and support them with ID.5 washers.

- Place the mounted ID.7 roller onto the M8 front bolt and ID.6 roller onto the rear M8 bolt.

- At the place where steel holder touches the frame or fairing, stick the 3M tape included in the kit.

- Finally, mount this assembly onto your motorcycle.

- Then, take the ID.8 bolt, M6 ID.10 washers and ID.9 nuts and mount them through the adapter and engine.

- Push the ID.13 strut above the engine and mount it onto the steel adapter. Use bolt and washer ID.14 and ID.15 table 3.

Mounting Instructions, Right Side:

- Remove the motorcycle fairing.

- At the place of a steel holder, remove the original bolts from motorcycle.

- Slip the ID.5 washers onto ID.4 bolts and push them through the ID.11 steel adapter.

- From rear side, mount ID.6 dural. rollers onto the bolts.

- Now, mount this assembly onto the motorcycle.

- Use the ID.15 bolt and ID. 14 washer to mount the ID.13 strut right above the engine table 3.

- Connect the adapter with the motorcylce with M6 ID.7 bolt, ID.9 washers and ID.8 nut.

- Finally, tighten all bolts on both sides.

Place back all fairing.

Mounting Instructions – Crash Protectors:

- Insert ID.3 bolt into the ID.2 adapter on the left side and place ID.1 roller underneath the adapter.

- Mount this assembly into the adapter on the frame.

- Mount the slider with ID.13 bolts and ID.14 nuts.

- Insert the ID.3 bolt into the ID.2 adapter on the right side and place ID.1 roller underneath the adapter.

- Mount this assembly into the adapter on the frame.

- Mount the slider using ID.12 bolts and ID.13 washers.

Follow the instructions shown on attached pictures during assembly.

Note: We recommend using blue color thread glue during assembly!

6

5

4

12

13

10

9

9, 7

9

7

8

11

4

5

4

6

5

8

6

10

5

14, 15

14, 15

13

4

13

12

L

R