K26

Kawasaki ZX250R-Ninja 2008-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M12 | X2 | 1 |  |  | 1 | Steel box M12 | X2 | 1 |  |
| 2 | Duralumin cylinder d=90mm | 238 | 1 |  |  | 2 | Duralumin cylinder d=60mm | 448 | 1 |  |
| 3 | Duralumin cylinder d=12mm | 233 | 1 |  |  | 3 | Duralumin cylinder d=46,5mm | 234 | 1 |  |
| 4 | Bolt M12x40mm (cone head) | 858 | 1 |  |  | 4 | Bolt M12x40mm (cone head) | 858 | 1 |  |
| 5 | Bold M12x40mm | 857 | 1 |  |  | 5 | Bolt M12x40mm | 857 | 1 |  |
| 6 | Silentblok D=30mm, d=15mm | X34 | 1 |  |  | 6 | Silentblok D=30mm, d=15mm | X34 | 1 |  |
| 7 | Poly nut d=23mm | 231 | 1 |  |  | 7 | Poly nut d=58mm | 232 | 1 |  |
| 8 | Dural.nut 22mm, d=33mm | 235 | 1 |  |  | 8 | Dural.nut 22mm, d=70mm | 236 | 1 |  |
| 9 | Bolt M10x30mm (cone head) | 846 | 2 |  |  | 9 | Bolt M10x30mm (cone head) | 846 | 2 |  |
| 10 | Nut M10-safety | 810 | 1 |  |  | 10 | Nut M10-safety | 810 | 1 |  |
| 11 | Pad M10-plane | 804 | 1 |  |  | 11 | Pad M10-plane | 804 | 1 |  |
| 12 | Duralumin adapter | 525 | 1 |  |  | 12 | Duralumin adapter | 526 | 1 |  |
| 13 | Steel bar M10x295x30x30 | - | 1 |  |  | 13 | 3M tape |  | 1 |  |
| 14 | 3M tape |  | 1 |  |  |  |  |  |  |  |

Installation:

- remove the original side plastic

- compile duralumin adapters according to the photos below

- remove the original long bolt M10, which hold the frame of the motorcycle and engine of the motorcycle

- give longer steel bolt into the frame of the motorcycle

- from the left and right side give duralumin adapters on this bolt

- mount adapters according to the photos below into the frame of the motorcycle and then use nuts M10 for tightening (give pads M10 under nuts)

- give steel box (No. 1) in protector together with bolt (No. 5)

- all this set mount to thread on adapter

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).