K29SD

Kawasaki ZX-10R (11-15)

| LEFT SIDE KIT |  |  |  |  |  |  |  | RIGHT SIDE KIT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description |  | Cat.no. | Pcs |  |  |  | ID |  | Description |  | Cat.no. |  | Pcs |  |
| 1 | SLD adapter |  | 537 | 1 |  |  |  | 1 |  | SLD adapter |  | 537 |  | 1 |  |
| 2 | Bolt M10x105x1,25 |  | 893 | 1 |  |  |  | 2 |  | Bolt M10x105x1,25 |  | 893 |  | 1 |  |
| 3 | Bolt M8x25 |  | 815 | 2 |  |  |  | 3 |  | Bolt M8x25 |  | 815 |  | 2 |  |
| 4 | Washer M8 |  | 800 | 2 |  |  |  | 4 |  | Washer M8 |  | 800 |  | 2 |  |
| 5 | Washer M10 (spring) |  | 801 | 1 |  |  |  | 5 |  | Washer M10 (spring) |  | 801 |  | 1 |  |
| 6 | Dural. cylinder 30,5mm |  | 290 | 1 |  |  |  | 6 |  | Dural. cylinder 30,5mm |  | 291 |  | 1 |  |
| 7 | Dural. cylinder 30mm |  | DV11 | 1 |  |  |  | 7 |  | Dural. cylinder 30mm |  | DV11 |  | 1 |  |

Instructions

- We recommend to use the glue on all threads.

- Dismantle original bolts from the frame.

- Put bolt ID2 through washer ID5, SLD adapter ID1 and dural cylinders ID6 and ID7 and fit this kit on the frame of motorcycle. Note: shorter cylinder ID7 is on the right side.

- Fit slider using two bolts ID3 and washers ID4.