K29S

Kawasaki ZX-10R (11-)

| Kit for the left side |  |  |  |  |  |  |  | Kit for the right side |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description |  | Cat.no. | Pcs |  |  |  | ID |  | Description |  | Cat.no. |  | pcs |  |
| 1 | Moto adapter |  | DV 321 | 1 |  |  |  | 1 |  | Moto adapter |  | DV 322 |  | 1 |  |
| 2 | Bolt M10x50x1,25mm |  | 921 | 1 |  |  |  | 2 |  | Bolt M10x70x1,25 |  | 898 |  | 1 |  |
| 3 | Bolt M6x25 |  | 877 | 3 |  |  |  | 3 |  | Bolt M6x25 |  | 877 |  | 3 |  |
| 4 | Bolt M4x16 |  | 876 | 2 |  |  |  | 4 |  | Bolt M4x16 |  | 876 |  | 2 |  |

Instructions:

- We recommend to use the glue on all threads (medium strong).

- Remove original bolt from frame of motorcycle.

- Use bolt (ID2) for tightening moto adapter (ID1) on the frame of motorcycle.

- Note: longer bolt with adapter is on the right side.

- Fit slider on the moto adapter using three bolts (ID3).

- Fit coloured cover using two bolts (ID4).

ID1

ID3

ID3

ID4