K2

Kawasaki ZX-6R 2003-2004

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Nut M10 locker | 810 | 1 |
| 3 | Bolt M10x140 | 953 | 1 |
| 4 | Dural.cylinder d=62mm | 151 | 1 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Nut M10 locker | 810 | 1 |
| 3 | Bolt M10x130 | 883 | 1 |
| 4 | Dural.cylinder d=20mm | 150 | 1 |

4

4

2

1

1

3

3

2

Fitting instruction left side:

- Into the protector insert ID.1 and after insert bolt ID.3.

- Put the dural.cylinder ID.4 under the protector.

- This kit mounting on the frame.

- Tighten bolt by nut ID.2.

Fitting instruction right side:

- Into the protector insert ID.1 and after insert bolt ID.3.

- Put the dural.cylinder ID.4 under the protector.

- This kit mounting on the frame.

- Tighten bolt by nut ID.2.

Warning: We recommend use the glue of all threads. Medium strong.