K30SDM

Kawasaki ER-6N ´12-

1

6

3

6

3

1

Kit for left side:

7

8

8

7

2

| ID. | Description | Cat.NO | QTY |
| --- | --- | --- | --- |
| 1 | Slider SLDM | SLDM | 1 |
| 2 | Bracket for SLDM | 537/B | 1 |
| 3 | Bolt M6x20 | 1024 | 2 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Nut M10 | 810 | 1 |
| 6 | Washer M6 | 1023 | 2 |
| 7 | Dural cylinder d=60mm | 14 | 1 |
| 8 | Steel adapter | 649 | 1 |
| 9 | Washer (spring) M10 | 801 | 1 |
| 10 | Bolt M10x90mm | 880 | 1 |

4,5 a 9

10

2

10

| Kit for right side: ID. | Description | Cat.No | QTY |
| --- | --- | --- | --- |
| 1 | Slider SLDM | SLDM | 1 |
| 2 | Bracket for SLDM | 537/B | 1 |
| 3 | Bolt M6x20 | 1024 | 2 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Nut M10 | 810 | 1 |
| 6 | Washer M6 | 1023 | 2 |
| 7 | Dural. cylinder d=60mm | 14 | 1 |
| 8 | Steel adapter | 648 | 1 |
| 9 | Washer spring M10 | 801 | 1 |
| 10 | Bolt M10x90mm | 880 | 1 |

Steel adapter for left side

Instruction to assembly left side:

- Dismantle original bolt from frame M10.

- On the frame mounting steel adapter (ID8) use original bolt.

- Put on the bolt M10x90 (ID10) bracket for SLDM (ID2) and dural. cylinder d=60mm (ID7), this set fit on the steel adapter and use washers and nut (ID4, ID9 and ID5).

- Fit on the bracket slider (ID1) use bolts and washers (ID6), (ID3).

Warning- use the glue of all threads. We recommended use medium strong glue.

Instruction to assembly right side :

- Dismantle original bolt from frame M10.

- On the frame mounting steel adapter (ID8) use original bolt.

- Put on the bolt M10x90 (ID10) bracket for SLDM (ID2) and dural. cylinder d=60mm (ID7), this set fit on the steel adapter and use washers and nut (ID4, ID9 and ID5).

- Fit on the bracket slider (ID1) use bolts and washers (ID6), (ID3).

Warning- use the glue of all threads. We recommended use medium strong glue.