K31SD

Kawasaki ER-6F 2012-

| Kit for left side: |  |  |  |  | Kit for right side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat.n. | Qty |  |  | ID. | Description | Cat.n. | Qty |  |
| 1 | Bracket for SLD | 537 | 1 |  |  | 1 | Bracket for SLD | 537 | 1 |  |
| 2 | Duralumin adapter | 9 | 1 |  |  | 2 | Duralumin adapter | 9 | 1 |  |
| 3 | Dural.cylinder d=40mm | 469 | 1 |  |  | 3 | Dural.cylinder d=40mm | 469 | 1 |  |
| 4 | Bolt M10x80 | 852 | 1 |  |  | 4 | Bolt M10x80 | 852 | 1 |  |
| 5 | Nut M10-safety | 810 | 1 |  |  | 5 | Nut M10-safety | 810 | 1 |  |
| 6 | Steel adapter | 639 | 1 |  |  | 6 | Steel adapter | 638 | 1 |  |
| 7 | Bolt M8x70 | 841 | 2 |  |  | 7 | Bolt M8x70 | 841 | 2 |  |
| 8 | Bolt M8x25 | 815 | 2 |  |  | 8 | Bolt M8x25 | 815 | 2 |  |
| 9 | Washer M8 | 802 | 2 |  |  | 9 | Washer M8 | 802 | 2 |  |
| 10 | 3M Tape | 3M | 1 |  |  | 10 | 3M Tape | 3M | 1 |  |

Instruct. to assembly- lesft side:

- Take out side fairings from bike. Dismantle bolts M8 from frame.

- Steel adapter ID.6 fitting on the frame,use bolts ID.7 (photo below).

- Bolt M10x80 ID.4 insert through the bracket ID.1,dural. reduction ID.2 and dural. cylinder ID.3.

- You can stick the 3M tape on the place, wher fairing is touching with adapter.

- This kit mounting on the steel adapter, from back side tighten bolt with nut ID.5.

- Fairing put back on the bike.

- Take bolts ID.7 and washers M8 ID.9 make a mounting sliders on the bracket.

Instruct. to assembly- right side:

- Take out side fairings from bike. Dismantle bolts M8 from frame.

- Steel adapter ID.6 fitting on the frame,use bolts ID.7 (photo below).

- Bolt M10x80 ID.4 insert through the bracket ID.1,dural. reduction ID.2 and dural. cylinder ID.3.

- You can stick the 3M tape on the place, wher fairing is touching with adapter.

- This kit mounting on the steel adapter, from back side tighten bolt with nut ID.5.

- Fairing put back on the bike.

- Take bolts ID.7 and washers M8 ID.9 make a mounting sliders on the bracket.

Warning – put the safety glue on all threads during the assembly. Use medium strong one.

Fitting kit SL01. Ilustration photo,for placet the gripping on the frame only.