K32S

Kawasaki Versys 1000 2012-2017

| Kit for left side: |  |  |  |  | Kit for right side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.n | Qt |  |  | ID | Description | Cat.n | Qt |  |
| 1 | Dural.adapter for slider | DV-344 | 1 |  |  | 1 | Dural.adapter for slider | DV-344 | 1 |  |
| 2 | Bolt M6x25 | 877 | 3 |  |  | 2 | Bolt M6x25 | 877 | 3 |  |
| 3 | Bolt M4x16 | 876 | 2 |  |  | 3 | Bolt M4x16 | 876 | 2 |  |
| 4 | Bolt M10x125x1,25mm | 924 | 1 |  |  | 4 | Bolt M10x125x1,25mm | 924 | 1 |  |
| 5 | Pad M10 | 801 | 1 |  |  | 5 | Pad M10 | 801 | 1 |  |

Fitting left side:

- Take ou original bolt from frame.

- Bolt ID.4 with pad ID.5 insert into the  adapter ID.1. This set mounting the place on frame.

- Place slider and tighten it by bolts ID.2.

- Place alu. cover into the slider use bolts ID.3.

Fitting right side:

- Take ou original bolt from frame.

- Bolt ID.4 with pad ID.5 insert into the  adapter ID.1. This set mounting the place on frame.

- Place slider and tighten it by bolts ID.2.

- Place alu. cover into the slider use bolts ID.3.

Warning - on the threads of the screws we recommend using glue medium strong(blue).