K32

Kawasaki Versys 1000 2012-2017

| Kit for left side: |  |  |  |  | Kit for right side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.n | Qt |  |  | ID | Description | Cat.n | Qt |  |
| 1 | Dural.adapter for slider | DV-20 | 1 |  |  | 1 | Dural.adapter for slider | DV-20 | 1 |  |
| 2 | Al. insert | X1 | 1 |  |  | 2 | Al. insert | X1 | 1 |  |
| 3 | Bolt M10x150x1,25mm | 932 | 1 |  |  | 3 | Bolt M10x150x1,25mm | 932 | 1 |  |
| 4 | Pad M10 | 801 | 1 |  |  | 4 | Pad M10 | 801 | 1 |  |

Fitting left and right side:

- Take original bolt from frame.

- Bolt ID.3 with pad ID.4 with AL insert ID. 2, insert into the protektor and adapter ID.1. This set mounting the place on frame.

Warning - on the threads of the screws we recommend using glue medium strong(blue).