K33SDM

Kawasaki Ninja 300 2013-

Kit for left side:						Kit for right side:

| ID. | Description | Cat.No | QTY |
| --- | --- | --- | --- |
| 1 | Slider | SLDM | 1 |
| 2 | Bracket for SLDM | 537/B | 1 |
| 3 | Bolt M6x20 | 1024 | 2 |
| 4 | Washer M6 | 1023 | 2 |
| 5 | Dural. redutc. d=15mm | DV-149 | 1 |
| 6 | Dural. cylinder d=70mm | 470 | 1 |
| 7 | Bolt M12x40 | 1021 | 1 |
| 8 | Bolt M10x60 | 850 | 2 |
| 9 | Nut M10 | 810 | 2 |
| 10 | Washer M10 | 804 | 4 |
| 11 | Steel adapter | 642 | 1 |
| 12 | Bolt M12x40 | 857 | 1 |

| ID. | Description | Cat.N | Ks |
| --- | --- | --- | --- |
| 1 | Slider | SLDM | 1 |
| 2 | Bracket for SLDM | 537/B | 1 |
| 3 | Bolt M6x20 | 1024 | 2 |
| 4 | Washer M6 | 1023 | 2 |
| 5 | Dural. redutc. d=15mm | DV-149 | 1 |
| 6 | Dural. cylinder d=70mm | 470 | 1 |
| 7 | Bolt M12x40 | 1021 | 1 |
| 8 | Bolt M10x60 | 850 | 2 |
| 9 | Nut M10 | 810 | 2 |
| 10 | Washer M10 | 804 | 4 |
| 11 | Steel adapter | 643 | 1 |
| 12 | Bolt M12x40 | 857 | 1 |

9,10

9,10

8

11

11

8

3,4

7

6

5

12

2

2

6

12

5

3,4

1

Instruction to assembly- left side:

- Take out fairing and original bolts M10 from frame.

- On the steel adapter ID.11 mounting dural.cylinder ID.6 use bolt ID.7.

- This set mounting on the frame use the washers ID.10, bolts ID.8 and nuts ID.9.

- Put fairing on the bike,now.

- Put on the bolt ID.12 bracket for SLD ID.2 and dural. reduction ID.5

- This set fixed on the dural. cylinder.

- Mounting slider ID.1 use the bolts and washers ID.3 and ID.4.

Warning- we recommend use the glue of all threads. Medium strong.

Instruction to assembly- right side:

- Take out fairing and original bolts M10 from frame.

- On the steel adapter ID.11 mounting dural.cylinder ID.6 use bolt ID.7.

- This set mounting on the frame use the washers ID.10, bolts ID.8 and nuts ID.9.

- Put fairing on the bike,now.

- Put on the bolt ID.12 bracket for SLD ID.2 and dural. reduction ID.5

- This set fixed on the dural. cylinder.

- Mounting slider ID.1 use the bolts and washers ID.3 and ID.4.

Warning- we recommend use the glue of all threads. Medium strong.