K33S

Kawasaki Ninja 300 2013-

Kit for left side:					   Kit for right side:

| ID. | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Steel adapter | 643 | 1 |
| 2 | Bolt M12x40 | 1021 | 1 |
| 3 | Bolt M10x60 | 850 | 2 |
| 4 | Dural. cylinder d=70 | 470 | 1 |
| 5 | Nut M10 | 810 | 2 |
| 6 | Washer M10 | 804 | 4 |
| 7 | Adapter for SL01 | 1137 | 1 |
| 8 | Bolt M12x40 | 857 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |

| ID. | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Steel adapter | 642 | 1 |
| 2 | Bolt M12x40 | 1021 | 1 |
| 3 | Bolt M10x60 | 850 | 2 |
| 4 | Dural.cylinder d=70 | 470 | 1 |
| 5 | Nut M10 | 810 | 2 |
| 6 | Washer M10 | 804 | 4 |
| 7 | Adapter for SL01 | 1137 | 1 |
| 8 | Bolt M12x40 | 857 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |

4

1

8

9

5

10

3

6

6

9

2

5

8

7

4

10

1

3

2

6

7

Instruction to assembly:

Assembly for left and right side is same.

Do the first left side and after do the right side.

- Take off fairings and bolts M10 from frame.

- Mount dural.cylinder ID.4,use bolt M12x40 ID.2 into the steel adapter ID.1.

- This set mounted on frame , use bolts ID.3 washers ID.6 and nuts ID.5.

- Put fairing back on the bike.

- The bolt M12 ID.8 insert into the adapter for  SL01 ID.7 and mounting it on the steel adapter.

- Use bolts M6x25 ID.9 for fixed sliders, insert cover and use bolts M4x16 ID.10

Warning – use the glue of all threads. Medium strong.