K33

Kawasaki Ninja 300 2013-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.No | Pcs |  |  | No. | Description | Cat.No |  | Pcs |  |
| 1 | steel insert M12 | X2 | 1 |  |  | 1 | steel insert M12 | X2 |  | 1 |  |
| 2 | screw M12x40mm | 1021 | 1 |  |  | 2 | screw M12x40mm | 1021 |  | 1 |  |
| 3 | duralumin cylinder d = 70mm | 470 | 1 |  |  | 3 | duralumin cylinder d = 70mm | 470 |  | 1 |  |
| 4 | screw M10x60mm | 850 | 2 |  |  | 4 | screw M10x60mm | 850 |  | 2 |  |
| 5 | steel adapter | 642 | 1 |  |  | 5 | steel adapter | 643 |  | 1 |  |
| 6 | nut M10-insurance | 810 | 2 |  |  | 6 | nut M10-insurance | 810 |  | 2 |  |
| 7 | pad M10 flat | 804 | 4 |  |  | 7 | pad M10 flat | 804 |  | 4 |  |
| 8 | screw M12x40 | 857 | 1 |  |  | 8 | screw M12x40 |  | 857 | 1 |  |

Mounting - left side:

- remove the original fairing side and screws M10 of the motorcycle frame

- from the back side of the steel adapter (item 5) insert bolt (item 2)and fixed duralumin cylinder (item 3)

- by screws (item 4), nuts (item 6) and pads (item 7) fixed on the motorcycle frame steel adapter (item 5)

- to the crash protector insert a steel insert (item 1) and screw (item 8)

- mounting the fairing on te bike

Warning - the threads of the screws is recommended to use premium adhesive moderate (blue color).

Mounting - right side:

- remove the original fairing side and screws M10 of the motorcycle frame

- from the back side of the steel adapter (item 5) insert bolt (item 2)and fixed duralumin cylinder (item 3)

- by screws (item 4), nuts (item 6) and pads (item 7) fixed on the motorcycle frame steel adapter (item 5)

- to the crash protector insert a steel insert (item 1) and screw (item 8)

- mounting the fairing on te bike

Warning - the threads of the screws is recommended to use premium adhesive moderate (blue color).