K34-2

Kawasaki ZX636R Ninja,´13-´14

Fitting kit left side:	 	     		     Fitting kit right side:

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Steel box | X1 | 1 |
| 2 | Dural.adapter | 1128 | 1 |
| 3 | Bolt M10x80x1,25 | 920 | 1 |

| ID | Description | Cat.n | QTY |
| --- | --- | --- | --- |
| 1 | Steel box | X1 | 1 |
| 2 | Dural.adapter | 1129 | 1 |
| 3 | Bolt M10x90x1,25 | 890 | 1 |

Instruction to assembly left side:

- Take out original bolt M10 from frame.

- Insert steel box ID.1 into the protector.

- Insert bolt ID.3 M10x80 into the protector and put on it dural.adapter ID.2.

- This set instal into the frame.

Instruction to assembly right side:

- Take out original bolt M10 from frame.

- Don´t dismantle special crown nut!

- Insert steel box ID.1 into the protector.

- Insert bolt ID.3 M10x80 into the protector and put on it dural.adapter ID.2.

- This set instal into the frame.

Warning : Use the glue of all threads-medium strong.

2

1

3

1

2

3