K35SD

Kawasaki Z800´013-

|  | Kit for left side |  |  |  | Kit for right side |
| --- | --- | --- | --- | --- | --- |
| No. | Description | Cat. No. | Quant. |  |  |
| - | ------ | - | - |  |  |
| 2 | Dural. adapter on the frame | 547 | 1 |  |  |
| 3 | Dural. cylinder d=20mm | 495 | 2 |  |  |
| 4 | Holder for SLD | 537 | 1 |  |  |
| 5 | Dural. reduction d=20mm | 10 | 1 |  |  |
| 6 | Bolt M8x25mm | 815 | 2 |  |  |
| 7 | Washer spring M8 | 800 | 2 |  |  |
| 8 | Bolt M8x70mm | 841 | 2 |  |  |
| 9 | Washer spring M10 | 801 | 1 |  |  |
| 10 | Bolt M10x45mm | 848 | 1 |  |  |
| 11 | Plastic cover - black | X5 | 2 |  |  |
| 12 | 3M sticker |  | 1 |  |  |

| No. | Description | Kat.No. | Quant. |
| --- | --- | --- | --- |
| - | ------- | - | - |
| 2 | Dural. adapter on the frame | 547 | 1 |
| 3 | Dural. cylinder d=20mm | 495 | 2 |
| 4 | Holder for SLD | 537 | 1 |
| 5 | Dural. reduction d=15mm | 9 | 1 |
| 6 | Bolt M8x25mm | 815 | 2 |
| 7 | Washer spring M8 | 800 | 2 |
| 8 | Bolt M8x70mm | 841 | 2 |
| 9 | Washer spring M10 | 801 | 1 |
| 10 | Bolt M10x40mm | 854 | 1 |
| 11 | Plastic cover - black | X5 | 2 |
| 12 | 3M sticker |  | 1 |

4

9,10

44

2

3

2

11

8

8

9,10

11

5

3

12

7

6

5

6,7

Instruction left side:

- Remove original bolt M8 from frame.

- Insert the frame cylinder (No.3) and mounting duralumin adapter on the frame (no.2) using bolt M8 (no.8).

- Mounting SLD holder (no.4 ) using duralumin reduciton (no.5),bolt M10 (poz.10) and washer (no.9) all kit fixed on the duralumin adapter.

- Using bolt M8x25 (no.6) and washer (poz.7) mounting slider SLD.

Insert plastic covero in to the bolt and stick 3M between the duralumin holde rand motorcycle frame.

Instruction- right side:

- Remove original bolt M8 from frame.

- Insert the frame cylinder (No.3) and mounting duralumin adapter on the frame (no.2) using bolt M8 (no.8).

- Mounting SLD holder (no.4 ) using duralumin reduciton (no.5),bolt M10 (poz.10) and washer (no.9) all kit fixed on the duralumin adapter.

- Using bolt M8x25 (no.6) and washer (poz.7) mounting slider SLD.

Insert plastic covero in to the bolt and stick 3M between the duralumin holde rand motorcycle frame.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).