K37S

Kawasaki Z1000 2014-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Adapter moto d=40mm | DV-317 | 1 |  |  | 1 | Adapter moto d=40mm | DV-317 | 1 |  |
| 2 | Bolt M6x25 | 877 | 3 |  |  | 2 | Bolt M6x25 | 877 | 3 |  |
| 3 | Bolt M4x16 | 876 | 2 |  |  | 3 | Bolt M4x16 | 876 | 2 |  |
| 4 | Duralumin adapter | 548 | 1 |  |  | 4 | Duralumin adapter | 549 | 1 |  |
| 5 | Duralumin adapter | 550 | 1 |  |  | 5 | Dural. cylinder d=19mm | 460 | 2 |  |
| 6 | Dural. cylinder d=8mm | 462 | 1 |  |  | 6 | Bolt M10x30mm | 882 | 1 |  |
| 7 | Dural. cylinder d=11mm | 461 | 1 |  |  | 7 | Bolt M10x100x1,25mm | 892 | 1 |  |
| 8 | Bolt M10x30mm | 882 | 1 |  |  | 8 | Bolt M10x110x1,25mm | 928 | 1 |  |
| 9 | Bolt M10x100x1,25mm | 892 | 1 |  |  | 9 | Plastic cover - black | X4 | 2 |  |
| 10 | Bolt M10x80x1,25mm | 920 | 1 |  |  | 10 | 3M tape |  | 1 |  |
| 11 | Bolt M6x30mm | 863 | 2 |  |  |  |  |  |  |  |
| 12 | Plastic cover - black | X4 | 2 |  |  |  |  |  |  |  |
| 13 | 3M tape |  | 1 |  |  |  |  |  |  |  |

Installation – the left side:

- remove the original bolts

- compile the duralumin adapter together, which has a two parts (No. 4 and No. 5), use bolts (No. 11) for mounting this two parts together

- if you have mounted the adapter, so insert the bolts (No. 9 and No. 10) into the adapter, give the duralumin cylinders (No. 6 and No. 7) on bolts

- all this set mount into the frame of the motorcycle, tighten and give the plastic covers (No. 12) on the head of bolts, use tape 3M for mounting in the place, where the adapter touches the frame of the motorcycle or the engine

- if you mounted duralumin cylinder, you can mount gripping for slider. Now use the bolt (No. 8) for mounting of duralumin cylinder (No. 1) on the duralumin adapter, you have to mount adapter with thread M6 up, then slider have the right rotation.

- put slider and use the bolts (No. 2) for mounting

- insert in slider the cover and use the bolts (No. 3) for gripping

Installation – the right side:

- remove the original bolts

- take the adapter (No.4), insert the bolts (No. 7 and No. 8) into the adapter, give the duralumin cylinders (No. 5) on bolts

- all this set mount into the frame of the motorcycle, tighten and give the plastic covers (No. 9) on the head of bolts, use tape 3M for mounting in the place, where the adapter touches the frame of the motorcycle or the engine

- if you mounted duralumin cylinder, you can mount gripping for slider. Now use the bolt (No.6) for mounting of duralumin cylinder (No. 1) on the duralumin adapter, you have to mount adapter with thread M6 up, then slider have the right rotation.

- put slider and use the bolts (No. 2) for mounting

- insert in slider the cover and use the bolts (No. 3) for gripping

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).