K38SD

Kawasaki Z 300 2015-

Kit for left side:					   Kit for right side:

| ID. | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Steel adapter | 705 | 1 |
| 2 | Alu spacer d=5mm | 16 | 2 |
| 3 | Bolt M10x60 | 850 | 2 |
| 4 | Washer M10 | 804 | 4 |
| 5 | Nut M10 | 810 | 2 |
| 6 | Alu. slider holder | 537 | 1 |
| 7 | Al. spacer len=30mm | 11 | 1 |
| 8 | Bolt M10x60 | 850 | 1 |
| 9 | Bolt M8x25 | 815 | 2 |
| 10 | Washer M8 | 800 | 2 |

| ID. | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Steel adapter | 704 | 1 |
| 2 | Alu spacer d=5mm | 16 | 2 |
| 3 | Bolt M10x60 | 850 | 2 |
| 4 | Washer M10 | 804 | 4 |
| 5 | Nut M10 | 810 | 2 |
| 6 | Alu. slider holder | 537 | 1 |
| 7 | Al. spacer len=30mm | 11 | 1 |
| 8 | Bolt M10x60 | 850 | 1 |
| 9 | Bolt M8x25 | 815 | 2 |
| 10 | Washer M8 | 800 | 2 |

7

6

3,4,5

7

2

3,4,5

8

8

6

Instruction to assembly:

Assembly for left and right side is same.

Make first left side and after done right side.

- Take off fairings and bolts M10 from frame.

- Mounting steel holder ID.1. Under the steel adapter place alu spacers ID.2. use bolts M10x60 ID.3, washers ID.4 and tighten it by nuts ID.5, unde the nuts place washers.

- Put fairing back on the bike.

- On the bolt ID.8 insert slider holder ID.6, under the slider holder place alu. spacer ID.7. This kit assmebly by bolt ID.8.

- Place slider and fix it by bolts and washers M8.

Warning – use the glue of all threads. Medium strong.

3,4

1

3,4

1

6