K38S

Kawasaki Z 300 2015-

Kit for left side:					   Kit for right side:

| ID. | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Steel adapter | 705 | 1 |
| 2 | Alu spacer d=5mm | 16 | 2 |
| 3 | Bolt M10x60 | 850 | 2 |
| 4 | Washer M10 | 804 | 4 |
| 5 | Nut M10 | 810 | 2 |
| 6 | Alu. slider holder | 317 | 1 |
| 7 | Bolt M10x35 | 847 | 1 |
| 8 | Bolt M6x25 | 877 | 3 |
| 9 | Bolt M4x16 | 876 | 2 |

| ID. | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Steel adapter | 704 | 1 |
| 2 | Alu spacer d=5mm | 16 | 2 |
| 3 | Bolt M10x60 | 850 | 2 |
| 4 | Washer M10 | 804 | 4 |
| 5 | Nut M10 | 810 | 2 |
| 6 | Alu. slider holder | 317 | 1 |
| 7 | Bolt M10x35 | 847 | 1 |
| 8 | Bolt M6x25 | 877 | 3 |
| 9 | Bolt M4x16 | 876 | 2 |

6

3,4,5

3,4,5

8

6

3,4,5

2

3,4,5

8

Instruction to assembly:

Assembly for left and right side is same.

Make first left side and after done right side.

- Take off fairings and bolts M10 from frame.

- Mounting steel holder ID.1. Under the steel adapter place alu spacers ID.2. use bolts M10x60 ID.3, washers ID.4 and tighten it by nuts ID.5, unde the nuts place washers.

- Put fairing back on the bike.

- The bolt M12 ID.7 insert into the adapter for  SL01 ID.6 mounting it on the steel adapter.

- Use bolts M6x25 ID.8 for fixed sliders, insert cover and use bolts M4x16 ID.9

Warning – use the glue of all threads. Medium strong.

1

3,4

3,4

1

6

2