K39S

Kawasaki Versys 650 2015-

| Kit for left side: Kit for right side: ID | info | Cat.no. | pcs |
| --- | --- | --- | --- |
| 1 | Dural. Adapter - 90mm | 346 | 1 |
| 2 | Nut M10 | X11 | 1 |
| 3 | Screw M6x25 | 877 | 3 |
| 4 | Scew M4x16 | 876 | 2 |

| ID | info | Cat.no. | pcs |
| --- | --- | --- | --- |
| 1 | Dural. adapter - 40mm | 317 | 1 |
| 2 | Nut M10 | X11 | 1 |
| 3 | Screw M6x25 | 877 | 3 |
| 4 | Screw M4x16 | 876 | 2 |
| 5 | Steel connecting rod M10x455x1,25x30x30 |  | 1 |

Mounting procedure is same on both sides. Diference is only in dimensions of adapters.

- Dismount original screws from frame of motorcycle.

- Leave silentblock and original steel washer on frame.

- Pass connecting rod M10 ID.5. through frame and engine.

- Put adapters ID.1 on connecting rod (90mm on the left and 40mm on the right). Tighten rod with nuts M10 ID.2.

- Fit slider on adapter with three screws ID.3.

- Fit cover on slider with two screws ID.2.

We recommend: use the glue on all threads. Medium strong.