K39

Kawasaki Versys 650 2014-

Kit for the left side:				kit for the right side:

| ID. | Description | Cat.n. | pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | X1 | 1 |
| 2 | Dural. adapter d=15mm | DV9 | 1 |
| 3 | Nut M10 | X11 | 1 |
| 4 | M10x525x1,25x30x30 | - | 1 |

| ID. | Description | Kat.č. | pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | X1 | 1 |
| 2 | Dural. adapter d=40mm | DV4 | 1 |
| 3 | Dural. adapter d=20mm | DV10 | 1 |
| 4 | Nut M10 | X11 | 1 |

Montážní postup:

- Remove the engine mounting bolts.

- Put connection rod M10 through frame.

- Put steel adapters ID.1. into protectors

- Left side: Place dural adapter ID.2 and dural reduction ID.3 under protector.

- After that place protector and screw M10.

- Right side: Push screw-thread through protector. Place dural reduction ID.2 under protector. Put the nut M10.

- Tighten the nuts.

Warning – use the glue of all threads. Medium strong.