K40S

Kawasaki Vulcan 650 2015-

| LEFT SIDE KIT |  |  |  |  | RIGHT SIDE KIT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pos. | Popis | Kat.číslo | Počet |  |  | Pos. | Popis | Kat.číslo | Počet |  |
| 1 | Bolt M6x25 | 877 | 3 |  |  | 1 | Bolt M6x25 | 877 | 3 |  |
| 2 | Bolt M10x70 | 851 | 1 |  |  | 2 | Bolt M10x70 | 851 | 1 |  |
| 3 | dural. SL adapter 91mm | DV346 | 1 |  |  | 3 | dural.SL adapter 91mm | DV346 | 1 |  |
| 4 | Washer M10 | 804 | 1 |  |  | 4 | Washer M10 | 804 | 1 |  |
| 5 | Safety nut M10 | 810 | 1 |  |  | 5 | Safety nut M10 | 810 | 1 |  |
| 6 | Spring washerM10 | 801 | 1 |  |  | 6 | Spring washer M10 | 801 | 1 |  |
| 7 | Bolt M8x60 | 838 | 1 |  |  | 7 | Bolt M8x60 | 838 | 1 |  |
| 8 | Dural. cylinder 10mm | 1230 | 2 |  |  | 8 | Dural. cylinder 10mm | 1230 | 2 |  |
| 9 | Washer M8 | 802 | 2 |  |  | 9 | Washer M8 | 802 | 2 |  |
| 10 | Bolt M8x40 | 828 | 1 |  |  | 10 | Bolt M8x40 | 828 | 1 |  |
| 11 | Dural. cylinder 12,5mm | 1232 | 1 |  |  | 11 | Dural. cylinder 17,5mm | 1231 | 1 |  |
| 12 | Bolt M5x25 | 1036 | 1 |  |  | 12 | Bolt M5x35 | 1035 | 1 |  |
| 13 | Bolt M6x16 | 1033 | 1 |  |  | 13 | 3M tape |  | 2 |  |
| 14 | Steel adapter | 719 | 1 |  |  | 14 | Steel adapter | 720 | 1 |  |
| 15 | Bolt M4x12 | 876 | 2 |  |  | 15 | Bolt M4x12 | 876 | 2 |  |

INSTRUCTIONS

- Dismantle side fairing on both sides.

- Unbolt original bolt M8, M6 and M5 on both sides.

- Put washer ID6 and SL adapter ID3 on bolt ID2. Fit this kit using nut ID5 and washer ID4. Fit this kit on the frame of motorcycle using bolt ID7 and ID10 with washers ID9 and cylinders ID8.

- In the end fit cylinder ID11 using bolt ID12 on the steel adapter.

- Check all connections and place fairing back on the bike.

We recommend to use safety glue on all threads (medium strong).