K40

Kawasaki Vulcan 650 2015-

| LEFT SIDE KIT |  |  |  |  | RIGHT SIDE KIT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No. | Pcs |  |  | ID | Description | Cat.No. | Pcs |  |
| 1 | Steel insert M10 | X1 | 1 |  |  | 1 | Steel insert M10 | X1 | 1 |  |
| 2 | Bolt M10x110 | 879 | 1 |  |  | 2 | Bolt M10x110 | 879 | 1 |  |
| 3 | Dural. cylinder 60mm | DV6 | 1 |  |  | 3 | Dural. cylinder 60mm | DV6 | 1 |  |
| 4 | Washer M10 | 804 | 1 |  |  | 4 | Washer M10 | 804 | 1 |  |
| 5 | Safety nut M10 | 810 | 1 |  |  | 5 | Safety nut M10 | 810 | 1 |  |
| 6 | Spring washer M10 | 801 | 1 |  |  | 6 | Spring washer M10 | 801 | 1 |  |
| 7 | Bolt M8x60 | 838 | 1 |  |  | 7 | Bolt M8x60 | 838 | 1 |  |
| 8 | Dural. cylinder 10mm | 1230 | 2 |  |  | 8 | Dural. cylinder 10mm | 1230 | 2 |  |
| 9 | Washer M8 | 802 | 2 |  |  | 9 | Washer M8 | 802 | 2 |  |
| 10 | Bolt M8x40 | 828 | 1 |  |  | 10 | Bolt M8x40 | 828 | 1 |  |
| 11 | Dural. cylinder 12,5mm | 1232 | 1 |  |  | 11 | Dural. cylinder 17,5mm | 1231 | 1 |  |
| 12 | Bolt M5x25 | 1036 | 1 |  |  | 12 | Bolt M5x35 | 1035 | 1 |  |
| 13 | Bolt M6x16 | 1033 | 1 |  |  | 13 | 3M Tape |  | 2 |  |
| 14 | Steel adapter L | 719 | 1 |  |  | 14 | Steel adapter R | 720 | 1 |  |

We recommend to use safety glue on all threads (medium strong).

LEFT INSTRUCTIONS

- Dismante side fairing and original bolts M8, M5 and M6 from the frame of motorcycle.

- Put washer ID6, steel insert ID1, crash protector and dural. cylinder ID3 on the bolt M10 (ID2). Fit this kit on the steel adater using nut ID5 and washer ID4.

- Fit steel adapter using bolt ID7 (front) and ID10 (rear). Place cylinder ID8 and washers ID9 on both bolts.

- Fit steel adapter using bolt ID12, cylinder ID11 and bolt ID13.

- Mount fairing back on the motorcycle.

RIGHT INSTRUCTIONS

- Dismante side fairing and original bolts M8, M5 and M6 from the frame of motorcycle.

- Put washer ID6, steel insert ID1, crash protector and dural. cylinder ID3 on the bolt M10 (ID2). Fit this kit on the steel adater using nut ID5 and washer ID4.

- Fit steel adapter using bolt ID7 (front) and ID10 (rear). Place cylinder ID8 and washers ID9 on both bolts.

- Fit steel adapter using bolt ID12 and cylinder ID11.

- Mount fairing back on the motorcycle.