K42

Kawasaki ZX-10R 2016-

| Left side kit |  |  |  |  | Right side kit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no | Pcs |  |
| 1 | Steel insert M10 | X1 | 1 |  |  | 1 | Steel insert M10 | X1 | 1 |  |
| 2 | Bolt M10x90x1,25mm | 890 | 1 |  |  | 2 | Bolt M10x95x1,25mm | 927 | 1 |  |
| 3 | Dural. cylinder 30,5mm | 290 | 1 |  |  | 3 | Dural. cylinder 14,5mm | 291 | 1 |  |

Instructions:

- We recommend to use the glue on all threads (medium strong).

- Remove original bolt from the frame of motorcycle.

- Put steel insert into protector and mount it on the motorcycle using bolt ID2 and cylinder ID3.