K44SDM

Kawasaki Ninja 650  2017-

| Left side kit: | Right side kit: |
| --- | --- |

| ID | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | SLD adapter | 537B | 1 |
| 2 | Dural. adapter 15mm | 9 | 1 |
| 3 | Dural. cylinder 50mm | 1226 | 1 |
| 4 | Bolt M10x200 | 889 | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |
| 6 | Washer M6 | 1023 | 2 |
| 7 | Safety nut M10 | 810 | 1 |

| ID | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | SLD adapter | 537B | 1 |
| 2 | Dural. adaptér 15mm | 9 | 1 |
| 3 | Dural.váleček d=85mm | 1225 | 1 |
| 4 | Bolt M10x200 | 889 | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |
| 6 | Washer M6 | 1023 | 2 |
| 7 | Safety nut M10 | 810 | 1 |

Instructions:

- Uscrew original bolt from the frame.

- Put SLD adapter, dural adapter and dural. cylinder on the bolt M10 (ID4).

- Mount this kit on the frame of motorcycle using safety nut M10 (from back side).

- Mount slider on the SLD adapter using two bolts and washers M6.

We recommend to use the glue on all threads.