K44S

Kawasaki Ninja 650 2017-

| Left side kit: |  |  |  |  |  | Right side kit: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  | ID |  | Description | Cat.no. | Pcs |  |  |
| 1 | Adapter | DV-318 | 1 |  | 1 |  | Adapter | DV-318 | 1 |  |  |
| 2 | Adapter | DV-4 | 1 |  | 2 |  | Adapter | DV-144 | 1 |  |  |
| 3 | Bolt M6X25 | 877 | 3 |  | 3 |  | Bolt M6X25 | 877 | 3 |  |  |
| 4 | Bolt M4X16 | 876 | 2 |  | 4 |  | Bolt M4X16 | 876 | 2 |  |  |
| 5 | Bolt M10x150 | 939 | 1 |  | 5 |  | Bolt M10x160 | 979 | 1 |  |  |
| 6 | Safety nut M10 | 810 | 1 |  | 6 |  | Safety nut M10 | 810 | 1 |  |  |

Instrutions:

- Unscrew original bolt from the frame of motorcycle according to the picture.

- Put both adapters ID1 and ID2 on the bolt M10.

Note: Longer bolt is on the right

- Mount this kit on the frame of motorcycle using safety nut M10.

- Mount slider on the adapter using three bolts M6.

- In the end mount coloured cover into slider using two bolts M4.

We recommend to use the glue on all threads.