K4S

Kawasaki Z750 2004-2006

Kawasaki Z1000 2003-2006

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Slider RDmoto | SL | 1 |  |  | 1 | Slider Rdmoto | SL | 1 |  |
| 2 | Cover | SLcap.. | 1 |  |  | 2 | Cover | SLcap.. | 1 |  |
| 4 | Adapter moto | DV-318 | 1 |  |  | 4 | Adapter moto | DV-318 | 1 |  |
| 7 | Bolt M6x25 | 877 | 3 |  |  | 7 | Bolt M6x25 | 877 | 3 |  |
| 8 | Bolt M4x16 | 876 | 2 |  |  | 8 | Bolt M4x16 | 876 | 2 |  |
| 9 | Duralumin adapter | 545 | 1 |  |  | 9 | Duralumin adapter | 546 | 1 |  |
| 10 | Bolt M10x140mm | 953 | 1 |  |  | 10 | Bolt M10x140mm | 953 | 1 |  |
| 11 | Bolt M12x25mm | 860 | 1 |  |  | 11 | Bolt M12x25mm | 860 | 1 |  |
| 12 | Dural. cylinder d=56mm | 202 | 1 |  |  | 12 | Dural. cylinder d=56mm | 202 | 1 |  |
| 13 | Nut M10 | 814 | 1 |  |  | 13 | Nut M10 | 814 | 1 |  |
| 14 | Bolt M10x20mm | 819 | 1 |  |  | 14 | Bolt M10x20mm | 819 | 1 |  |
| 15 | Dural. bar d=377mm,M12 | AL20 | 1 |  |  | 15 | Plastic cover M10 | X4 | 1 |  |
| 16 | Plastic cover M10 | X4 | 1 |  |  | 16 | Tape 3M |  | 1 |  |
| 17 | Tape 3M |  | 1 |  |  |  |  |  |  |  |

Installation:

- remove the original bolts M10

- use the tape 3M during the installation of the duralumin adapter, stick it on the reverse side of the duralumin adapter

- use the bolt (No. 10) for mounting of the duralumin adapter (No. 9) on the left side, the other side of the duralumin adapter give the duralumin cylinder (No. 12) on the bolt M10, this cylinder defines the will between the frame of the motorcycle and the steel adapter

- use the nut M10 (No. 13) for tightening of the duralumin adapter on the left side

- use the bolt (No. 10) for mounting of the duralumin adapter (No. 9) on the right side, the other side of the duralumin adapter give the duralumin cylinder (No. 12) on the bolt (No. 10)

- use the nut M10 (No. 13) for tightening of the duralumin adapter on the right side

- take the duralumin bar (No. 15, AL7), use the bolts (No. 11) for the gripping of the bar on the duralumin adapters

- if you mounted duralumin adapters, you can screw up it

- you can mount gripping for slider on the left side. Use bolt (No. 14) for mounting of duralumin adapter (No. 4) on the duralumin adapter. You have to mount adapter with thread M6 up, then slider have the right rotation.

- put slider (No. 1) and use the bolts (No. 7) for mounting

- insert in slider the cover (No. 2) and use the bolts (No. 8) for gripping

- use bolt (No. 14) for mounting of duralumin adapter (No. 4) on the duralumin adapter on the right side. You have to mount adapter with thread M6 up, then slider have the right rotation.

- put slider (No. 1) and use the bolts (No. 7) for mounting

- insert in slider the cover (No. 2) and use the bolts (No. 8) for gripping

- give the plastic cover (No. 16 and No. 15) on the head of bolts M10

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).