K50SD

Kawasaki Ninja H2 SX SE  2018-

Kit for the Left Side:		     		 Kit for the Right Side:

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. SLD holder | 537 | 1 |
| 2 | Dural. adapter | 12 | 1 |
| 3 | Dural. sleeve length =72mm | 201 | 1 |
| 4 | Bolt M10x150x1,25 | 932 | 1 |
| 5 | Washer M8 | 800 | 2 |
| 6 | Bolt M8x25 | 815 | 2 |

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Dural. SLD holder | 537 | 1 |
| 2 | Dural. adapter | 12 | 1 |
| 3 | Dural. sleeve l=52mm | 200 | 1 |
| 4 | Bolt M10x130x1,25 | 905 | 1 |
| 5 | Washer M8 | 800 | 2 |
| 6 | Bolt M8x25 | 815 | 2 |

Mounting- Left Side:

- Remove original bolt from the frame.

- Insert the bolt ID 4 into the dural. holder ID 1 + adalter ID 2 and dural. sleeve ID 3. Mount the whole assembly to the motorcycle frame.

- Place the slider on the plate and mount it using bolts ID 6 and washer ID 5.

Mounting- Right Side:

- Remove original bolt from the frame.

- Insert the bolt ID 4 into the dural. holder ID 1 + adalter ID 2 and dural. sleeve ID 3. Mount the whole assembly to the motorcycle frame.

- Place the slider on the plate and mount it using bolts ID 6 and washer ID 5.

We recommend using medium strenght thread glue during assembly.