K50S

Kawasaki Ninja H2 SX SE 2018-

| Mounting Kit - Left Side: |  |  |  |  | Mounting Kit - Right Side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No | Pcs |  |  | ID | Description | Cat.No | Pcs |  |
| 1 | Dural. adapter l=40mm | DV-317 | 1 |  |  | 1 | Dural. adapter l=40mm | DV-317 | 1 |  |
| 2 | Dural. sleeve l=72mm | 201 | 1 |  |  | 2 | Dural. sleeve l=52mm | 200 | 1 |  |
| 3 | Bolt M6x25 | 877 | 3 |  |  | 3 | Bolt M6x25 | 877 | 3 |  |
| 4 | Bolt M4x16 | 876 | 2 |  |  | 4 | Bolt M4x16 | 876 | 2 |  |
| 5 | Bolt M10x115x1,25 | 894 | 1 |  |  | 5 | Bolt M10x95x1,25 | 927 | 1 |  |

Mounting instructions – Left Side:

- Remove original bolt according to the picture below.

- Attach the dural. adapter ID.1 and ID.2 to the motorcycle frame using the bolt ID.5.

- Attach the slider and fasten it using bolts ID.3.

- Insert the cover into the slider and fasten it using bolts ID.4.

Mounting instructions – Right Side:

- Remove original bolt according to the picture below.

- Attach the dural. adapter ID.1 and ID.2 to the motorcycle frame using the bolt ID.5

- Attach the slider and fasten it using bolts ID.3.

- Insert the cover into the slider and fasten it using bolts ID.4.

Warning: We recommend using medium strength (blue) thread glue during assembly.