K5

Kawasaki ZX-9R (2002-)

| Left side kit |  |  |  |  | Right side kit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description |  | Cat.no. | Pcs |  |
| 1 | Steel insert | X1 | 1 |  |  | 1 | Steel insert |  | X1 | 1 |  |
| 2 | Dural. cylinder – 15mm | DV144 | 1 |  |  | 2 | Dural cylinder - 15mm |  | DV144 | 1 |  |
| 3 | Bolt M10x110mm | 879 | 1 |  |  | 3 | Bolt M10x110mm |  | 879 | 1 |  |
| 4 | Nut M10 | 814 | 1 |  |  | 4 | Nut M10 |  | 814 | 1 |  |

ID4

ID2

ID1

ID3

Instructions:

- Remove original bolt from the frame of motorcycle according to the picture above.

- Insert steel insert (ID1) and bolt (ID3) into protector.

- Put cylinder ID2 on the bolt (cylinder should be between protector and frame of motorcycle).

- Mount this kit into the frame using nut M10 (ID4).

We recommend to use the glue on all threads (medium strong).