K7S

Kawasaki ER-6N 2006-2008

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Slider adapter | DV-313 | 1 |  |  | 1 | Slider adapter | DV-313 | 1 |  |
| 2 | Bolt M6x25 | 877 | 3 |  |  | 2 | Bolt M6x25 | 877 | 3 |  |
| 3 | Bolt M4x16 | 876 | 2 |  |  | 3 | Bolt M4x16 | 876 | 2 |  |
| 4 | Bolt M10x130x1,25mm | 905 | 1 |  |  | 4 | Bolt M10x120x1,25mm | 895 | 1 |  |
| 5 | Bolt M8x60mm | 838 | 1 |  |  | 5 | Bolt M8x40mm | 835 | 2 |  |
| 6 | Bolt M10x25mm | 868 | 1 |  |  | 6 | Bolt M10x25mm | 868 | 1 |  |
| 7 | Dural. cylinder 63mm | 118 | 1 |  |  | 7 | Dural. cylinder 4,5mm | 116 | 1 |  |
| 8 | Dural. cylinder 22mm | 117 | 1 |  |  | 8 | Duralumin adapter | 527 | 1 |  |
| 9 | Duralumin adapter | 528 | 1 |  |  | 9 | Pad M10 (plane) | 804 | 1 |  |
| 10 | Pad M10 (plane) | 804 | 1 |  |  |  |  |  |  |  |

ID1+6

ID1+6

ID2

ID3

ID2

ID3

ID4+7+10

ID4+7+9

ID5+8

ID5

ID5

ID8

ID9

Instructions (left side):

- Dismantle side fairing.

- Fit steel adapter ID9 on the frame using two bolts (ID4+7+10 and ID5+8) according to the picture above.

- Fit slider adapter ID1 using M10 bolt ID6 into the steel adapter.

- In the end mount slider using three bolts (ID2) and coloured cover using two bolts (ID3).

We recommend to use the safety glue on all threads.

Instructions (right side):

- Dismantle side fairing.

- Fit steel adapter ID8 on the frame using two bolts (ID4+7+9 and two ID5) according to the picture above.

- Fit slider adapter ID1 using M10 bolt (ID6) into the steel adapter.

- In the end mount slider using three bolts (ID2) and coloured cover using two bolts (ID3).

We recommend to use the safety glue on all threads.