KTM10

KTM 390 Duke 2018-

| Kit for left side |  |  |  |  | Kit for right side |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat. No. | Pcs. |  | ID. | Description | Cat. No. | Pcs. |  |
| 1 | Steel insert M10 | X1 | 1 |  | 1 | Steel insert M10 | X1 | 1 |  |
| 2 | Bolt M10x75mm | 1007 | 1 |  | 2 | Bolt M10x75mm | 1007 | 1 |  |
| 3 | Dural. Roller d=13mm | 464 | 1 |  | 3 | Dural. Roller d=17mm | 1190 | 1 |  |
| 4 | Dural. Roller d=15mm | 465 | 1 |  | 4 | Dural. Roller d=17mm | 1190 | 1 |  |
| 5 | Dural. Roller d=40mm | 4 | 1 |  | 5 | Dural. Roller d=40mm | 4 | 1 |  |
| 6 | Bolt M8x45mm | 964 | 2 |  | 6 | Bolt M8x45mm | 964 | 2 |  |
| 7 | Steel Adapter | 628 | 1 |  | 7 | Steel Adapter | 628 | 1 |  |
| 8 | Washer M10 spring | 801 | 1 |  | 8 | Washer M10 spring | 801 | 1 |  |
|  |  |  | 3 3 |  |  | 7 7 |  |  |  |

6

6

1

1

5

5

4

4

6

6

2

2

8

8

Mounting Instructions:

- Mounting procedure for the protectors is the same for both sides.

- Take off the original M8 bolts.

- Insert the dural. rollers into the frame (ID.3 upper, ID. 4 lower).

- Use the steel ID.6 bolts to mount the ID.7 steel adapter to the motorcycle frame.

- Put the ID.1 steel insert into the protector together with ID.2 bolt and ID.8 washer.

- Take this assembly and mount it into the adapter thread.

Please note – We recommend using medium strenght glue for threads of all screws (blue color).