KTM4

KTM 390 Duke 2013-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M10 | X1 | 1 |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M10x75mm | 1007 | 1 |  | 2 | Bolt M10x75mm | 1007 | 1 |  |
| 3 | Dural. cylinder d=13mm | 464 | 1 |  | 3 | Dural. cylinder d=13mm | 464 | 1 |  |
| 4 | Dural. cylinder d=15mm | 465 | 1 |  | 4 | Dural. cylinder d=15mm | 465 | 1 |  |
| 5 | Dural. cylinder d=40mm | 4 | 1 |  | 5 | Dural. cylinder d=40mm | 4 | 1 |  |
| 6 | Bolt M8x45mm | 964 | 2 |  | 6 | Bolt M8x45mm | 964 | 2 |  |
| 7 | Steel adapter | 628 | 1 |  | 7 | Steel adapter | 628 | 1 |  |
| 8 | Washer M10-spring | 801 | 1 |  | 8 | Washer M10-spring | 801 | 1 |  |

3

8

7

6

6

5

4

2

1

Installation:

- the installation of crash protectors for the left and right side is the same

- remove the original bolts M8

- give duralumin cylinder (No. 3 UP, No. 4 Downl) into the frame

- use the bolts (No. 6) for gripping of steel adapter (No. 7) to the frame of the motorcycle

- give steel box (No. 1) in protector together with bolt (No. 2) and washer No.8 under the protector put dural. cylinder No.5

- all this set mount to thread on adapter

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).