MV1

MV Agusta F800 Brutale/Rivale 800

Mounting kit:

| No. | Description | Cat. no | Quant. |
| --- | --- | --- | --- |
| 1 | Steel nut M12 | X12/X2 | 2 |
| 2 | Dural. adapter | 493 | 2 |
|  | Steel barM12x430x1,25mm | N22 | 1 |
| 3 | Washer | 803 | 2 |

Instruction:

1.	Remove original bolt from frame.

2.	Through the frame steel bar (M12x430x1,25mm).

3.	Insert to the frame washer (note 3) and after insert duralumin adapter ( note 2).

4.	Fixed by nut (note 1).

Warning - the threads of the screws is recommended to use glue (blue color).

Montážní foto: