MV4

MV Agusta Brutale 990/ 1090

| Fitting kit for the left side |  |  |  |
| --- | --- | --- | --- |
| ID | Description | Cat.N | Qt |
| 1 | Steel insert | X2 | 1 |
| 2 | Screw M12x95x1.25 | 900 | 1 |
| 3 | Dural roller | 1204 | 1 |

| Fitting kit for the righ side |  |  |  |
| --- | --- | --- | --- |
| ID | Description | Cat.N | Qt |
| 1 | Steel insert | X2 | 1 |
| 2 | Screw M12x95x1.25 | 900 | 1 |
| 3 | Dural roller | 1204 | 1 |

Instruction to assembly:

We recommend to finish one side and than the second one.

- Remove original screws from frame of motorcycle.

- Put steel insert ID1 on screw ID2.

- Through dural roller ID3 tighten protector with screw ID2.

Warning - use the glue on all threads (medium strong)

ID2

ID3

ID1

Illustrative picture