Cagiva Navigator

Fitting kit left side:		     		  Fitting kit right side:

| ID | Description | Cat.No | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Screw M10x150x1,25 |  | 1 |
| 3 | Dural.cylinder 68mm |  | 1 |

| ID | Description | Cat.No | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Screw M10x135x1,25 |  | 1 |
| 3 | Dural.cylinder 77mm |  | 1 |

Assembly make to the frame, position on photos. Make first left side.

Assembly to left side:

- Take out original bolt from frame.

- Insert steel box ID.1 into the protector and after insert bolt ID.2.

- Take dural.cylinder ID.3 and insert it on the bolt.

- This kit mount into the frame.

- Thighten the safety bolt on frame.

Assembly to right side:

- Take out original bolt from frame.

- Insert steel box ID.1 into the protector and after insert bolt ID.2.

- Take dural.cylinder ID.3 and insert it on the bolt.

- This kit mount into the frame.

Warning:

We recommend use the glue of all threads. Medium strong.