S13

Suzuki GSX-R 1000 2005-2006

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x90x1,25 | 890 | 1 |
| 3 | Dural.cylinder d=37 | 66 | 1 |

2

1

3

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x90x1,25 | 890 | 1 |
| 3 | Dural.cylinder d=27,5 | 65 | 1 |

3

1

2

Instruction to assembly left side:

- Take out orig. bolt from frame M10.

- Into the protector insert steel box ID.1 and after insert bolt ID.2.

- From back side put dural cylinder ID.3 on the bolt.

- This set mount on the frame. Replace orig. bolt.

Instruction to assembly right side:

- Take out orig. bolt from frame M10.

- Into the protector insert steel box ID.1 and after insert bolt ID.2.

- From back side put dural cylinder ID.3 on the bolt.

- This set mount on the frame. Replace orig. bolt.

Warning: We recommend use the glue of all threads. Medium strong.