S17

Suzuki GSF600/GSX750 Inazuma ´95-´04

Suzuki GSF1200/GSX1200 Inazuma ´96-´05

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no | Pcs |  |  | No. | Description | Cat.no | Pcs |  |
| 1 | Steel box M12 | X2 | 1 |  |  | 1 | Steel box M12 | X2 | 1 |  |
| 2 | Bolt M12x40mm | 857 | 1 |  |  | 2 | Bolt M12x40mm | 857 | 1 |  |
| 3 | Dural. cylinder d=47mm | 48 | 1 |  |  | 3 | Dural. cylinder d=47mm | 48 | 1 |  |
| 4 | Dural. cylinder d=50mm | 283 | 1 |  |  | 4 | Dural. cylinder d=40mm | 282 | 1 |  |
| 5 | Dural. cylinder d=13mm | 86 | 1 |  |  | 5 | Dural. cylinder d=13mm | 86 | 1 |  |
| 6 | Bolt M10x120mm | 881 | 1 |  |  | 6 | Bolt M10x120mm | 881 | 1 |  |
| 7 | Bolt M8x80mm | 842 | 2 |  |  | 7 | Bolt M8x80mm | 842 | 2 |  |
| 8 | Bolt M12x30mm | 855 | 1 |  |  | 8 | Bolt M12x30mm | 855 | 1 |  |
| 9 | Pad M10 (plane) | 804 | 1 |  |  | 9 | Pad M10 (plane) | 804 | 1 |  |
| 10 | Nut M10 | 814 | 1 |  |  | 10 | Nut M10 | 814 | 1 |  |
| 11 | Duralumin adapter | 558 | 1 |  |  | 11 | Duralumin adapter | 559 | 1 |  |

Installation – the left side:

- remove the original bolts M10 before the engine and two bolts M8, which connect the holder and the frame of motorcycle

- use the bolt (No. 8) for mounting of duralumin cylinder (No. 4) on adapter (No. 11), give the bolt on reverse side of adapter

- installation of adapter on motorcycle is not difficult. At first give the bolt (No. 6) with pad (No. 9) and bolts (No. 7) in adapter. On reverse side of adapter give on bolst M10 the duralumin cylinder (No. 3) and on top bolt M8 the duralumin cylinder (No. 5). Mount all carefully on the motorcycle.

- put steel box (No. 1) together with bolt (No. 2) in protector

- all this set mount to thread of duralumin cylinder on adapter

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).

Installation – the right side:

- remove the original bolts M10 before the engine and two bolts M8, which connect the holder and the frame of motorcycle

- use the bolt (No. 8) for mounting of duralumin cylinder (No. 4) on adapter (No. 11), give the bolt on reverse side of adapter

- installation of adapter on motorcycle is not difficult. At first give the bolt (No. 6) with pad (No. 9) and bolts (No. 7) in adapter. On reverse side of adapter give on bolst M10 the duralumin cylinder (No. 3) and on top bolt M8 the duralumin cylinder (No. 5). Mount all carefully on the motorcycle.

- give steel box (No. 1) together with bolt (No. 2) in protector

- all this set mount to thread of duralumin cylinder on adapter

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).