S19*

Suzuki TL1000 R 1998-

Kit for left side :		     		   Kit for right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x135x1,25 | 930 | 1 |
| 3 | Alu spacer d=74 | 240 | 1 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M125x1x1,25 | 924 | 1 |
| 3 | Alu spacer d=64 | 239 | 1 |

2

1

3

1

3

2

*It´s necessary to dril hole into the fairing.

Done left side first and after finishing make other side.

Left side instruction:

- Take off original bolt from chassi.

- Drill hole into the fairing diameter 30,5 mm

- Insert steel box ID.1 intoprotector and after insert bolt ID.2.

- Under the protector place dural. spacer ID.3.

- This kit mount on the chassi.

Right side instruction:

- Take off original bolt from chassi.

- Drill hole into the fairing diameter 30,5 mm

- Insert steel box ID.1 intoprotector and after insert bolt ID.2.

- Under the protector place dural. spacer ID.3.

- This kit mount on the chassi.

During assembly place Locktite glue on the threads.