S20S

Suzuki GSR600 2006-

| Left side kit |  |  |  |  | Right side kit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No. | Pcs |  |  | ID | Description | Cat.No. | Pcs |  |
| 1 | SL adapter | 317 | 1 |  |  | 1 | SL adapter | 317 | 1 |  |
| 2 | Bolt M10x115x1,25mm | 894 | 1 |  |  | 2 | Bolt M10x85x1,25mm | 934 | 1 |  |
| 3 | Dural. cylinder 49mm | 43 | 1 |  |  | 3 | Dural. cylinder 34mm | 40 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |

Instructions:

- Remove original bolts from the frame according to the picture below.

- Put SL adapter and dural. cylinder in order according to the picture below on the bolt M10 (ID2).

- Mount this kit into the frame of motorcycle.

- Fit slider on the slider adapter using three bolts M6.

- Fit coloured cover using two bolts M4.

We recommend to use the glue on al threads (medium strong).