S23SDM

Suzuki GSX-R 600/750 2008-2010

| Kit for the left side: | Kit for the right side: |
| --- | --- |

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | SLDM adapter | 537/B | 1 |
| 2 | Bolt M10x60x1,25mm | 896 | 1 |
| 3 | Dural. spacer d=5mm | 16 | 1 |
| 5 | Bolt M8x25mm | 832 | 2 |
| 6 | Poly nut d=20mm | X9 | 2 |
| 7 | Duralumin adapter | 561 | 1 |
| 8 | Tape 3M |  | 1 |
| 9 | Bolt M12x40 cone | 858 | 1 |
| 10 | Washer M12 | 803 | 1 |
| 11 | Dural cylinder – 70mm | 412 | 1 |
| 12 | Bolt M12x30 | 856 | 1 |
| 13 | Bolt M6x20 | 1024 | 2 |
| 14 | Washer M6 | 1023 | 2 |

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | SLDM adapter | 537/B | 1 |
| 2 | Bolt M10x60x1,25mm | 896 | 1 |
| 3 | - | - | - |
| 5 | Bolt M8x25mm | 832 | 2 |
| 6 | Poly nut d=20mm | X9 | 2 |
| 7 | Duralumin adapter | 560 | 1 |
| 8 | Tape 3M |  | 1 |
| 9 | Bolt M12x30 cone | 855 | 1 |
| 10 | Washer M12 | 803 | 1 |
| 11 | Dural cylinder - 40mm | 282 | 1 |
| 12 | Bolt M12x20 | 992 | 1 |
| 13 | Bolt M6x20 | 1024 | 2 |
| 14 | Washer M6 | 1023 | 2 |

ID1

ID5+6

ID7

ID2

ID3

ID8

ID5+6

ID11

ID13+14

ID10+ID9

ID12

Installation:

- We recommend to use safety glue on threads (medium strong).

- Remove fairing of the motorcycle.

- Remove original bolt M10 from the frame.

- Build the kit for protectors according to the picture above.

- Note1: between ID7 and ID11 place washer ID10.

- Note2: on the left side place spacer ID3 on bolt ID2.

- Note3: before tightening stick 3M tape on the frame.

- Put fairing back on the motorcycle.