S26

Suzuki GSX-650F 2008-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel box M10 | X1 | 1 |  |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M8x30mm | 834 | 2 |  |  | 2 | Bolt M10x55x1,25mm | 915 | 2 |  |
| 3 | Dural. cylinder d=70mm | 7 | 1 |  |  | 3 | Dural. cylinder d=10mm | 1 | 1 |  |
| 4 | Bolt M10x120mm | 881 | 1 |  |  | 4 | Dural. cylinder d=10mm | 111 | 2 |  |
| 5 | Pad M8 (spring) | 800 | 2 |  |  | 5 | Bolt M10x60mm | 850 | 1 |  |
| 6 | Steel adapter | 565 | 1 |  |  | 6 | Pad M10 (spring) | 801 | 2 |  |
| 7 | Tape 3M |  | 1 |  |  | 7 | Steel adapter | 564 | 1 |  |
| 8 | Dural. bar AL16(M10)x435x30x30 |  | 1 |  |  | 8 | Tape 3M |  | 1 |  |

Installation:

- remove the complete sice cowl and the bolts in the place of the gripping of steel adapters

- use the tape 3M during the installation of the steel adapters for the defining the place between the adapter and the frame of the motorcycle

- mount the steel adapters on the left and right side of the frame of the motorcycle according to the below attached pictures

- take the right adapter and give the bolt (No. 2) together with the pad (No. 6) into the adapter, the other side of the adapter give the duralumin cylinders (No. 4) on the bolts, screw up the adapter lightly

- use the bolt (No. 2) together with the pads (No. 5) for the gripping of the left adapter, screw up lightly

- give the steel box (No. 1) together with bolt (No. 4)  into the protector, give the duralumin cylinder (No. 3) under the protector, the cylinder give on bolt too

- all this set put through the steel adapter and mount into the duralumin bar (No. 8, AL5)

- do installation on the other side the same

- check all and screw up

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).