S27

Suzuki GSX 1250 Bandit ´07-

Fitting kit left side:		     		       Fitting kit right side:

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Bolt M8x40 | 828 | 2 |
| 2 | Alu.frame adapter | 568 | 1 |
| 3 | Special alu spacer | 187 | 1 |
| 4 | Alu. black adapter | 566 | 1 |
| 5 | Bolt M8x20 | 823 | 2 |
| 6 | Bolt M12x40 | 858 | 1 |
| 7 | Bolt M8x30 | 834 | 1 |
| 8 | Washer M8 | 802 | 1 |
| 9 | Al.spa. under protec | 283 | 1 |
| 10 | Steel box | X2 | 1 |
| 11 | Bolt M12x40 | 857 | 1 |

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Spec. Alu. spacer | 186 | 1 |
| 2 | Bolt M10x30 | 882 | 1 |
| 3 | Plastic cover | X4 | 1 |
| 4 | Steel U stirrup | 950 | 1 |
| 5 | Nut M8 | 951 | 2 |
| 6 | Washer M8 | 802 | 2 |
| 7 | Bolt M12x40 | 858 | 1 |
| 8 | Al.spa. under protec | 283 | 1 |
| 9 | Bolt M12x40 | 857 | 1 |
| 10 | Steel box | X2 | 1 |
| 11 | Alu. black adapter | 567 | 1 |
| 12 | 3M Tape | 3M | 1 |

1

5,6

5

4

5

2,3

11

1

2

9

7

11

10

8

10

4

9

3

7, 8

5,6

1

2

6

1

Instruction to assembly-left side:

- Dismantle original bolt M8 from frame.

- Insert bolts ID.1 into the adapter ID.2. This adapter assembly on the frame.

- Take bolt ID.7 and washer ID.8 and mount alu.spacer ID.3 from back side on the black alu. adapter.

- Place bolt cone M12x40 ID.6 from back side and assembly ID.9.

- By bolts ID.5 assembly adapter ID.4 on the frame adapter.

- Insert steel box into the protector ID.10 and assemby protector by bolt ID.11.

Instruction to assembly-right side:

- Dismantle nut from frame in fron of the engine. Assebmly ID.1 on the thread.

And tighte it by wrench 19mm.

- Place bolt cone M12x40 ID.7 from back side and assembly ID.8.

- Stick 3M tape on the chassis, place stirrup ID.4 andtighten it easy by nuts and washers ID.5, ID.6.

- Tighten bolt ID.2 with alu. spacer ID.1 on the top. Place plastic cover ID.3. Tighten stirrup.

- Insert steel box into the protector ID.10 and assemby protector by bolt ID.9.

Warning: We recommend use the glue of all threads. Medium strong.