S30SDM

Suzuki SFV 650 Gladius 2008-

Left side kit:

| Note | Description | No | Quant. |
| --- | --- | --- | --- |
| 1 | Washer M6 | 1023 | 2 |
| 2 | Aluminium adapter | 27 | 1 |
| 3 | Aluminium plate SLDM | 537/B | 1 |
| 4 | Nut M12 | 811 | 1 |
| 5 | bolt M6x20 | 1024 | 2 |
| 6 | Stainless bar 12x445x30x30 |  | 1 |

Right side kit:

| Note | Description | No. | Quant. |
| --- | --- | --- | --- |
| 1 | Washer M6 | 1023 | 2 |
| 2 | Aluminium adapter | 27 | 1 |
| 3 | Aluminium plate SLDM | 537/B | 1 |
| 4 | Nut M12(samojistící) | 811 | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |

For connecting of the left and right side use the stainless steel bar (note. 6) with a length of 445 mm, at both ends of the threads M12x1,5mm, which is included mounting kit.

During the installation is necessary to make a hole to plastic cover ont the frame.

Installation:

- remove the plastic cover on the frame

- remove original bolt from frame

- insert stainless bar (note 6)

- maket he hole to plastic cover

- put on the frame plstic cover with hole

- put on aluminium adapter (note 2) on the steel bar

- put on the adapter aluminium plate (note 3) and fixed by nut  (not 4)

- Mounting slider on the plate, by bol (note 5) and washers note 1.

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).

Photogallery:

6

5

5

4

4

3

3

2

2

1

1